# How to install the S25 modem

To install the C35i, the M35i or the S35i as a modem, you need the P35modem.inf file included in the zip archive you just downloaded. Please store the file on floppy disk or hard drive.

Double-click the "Modems" icon in the "Control panel"

## Step1

*we apologize, this document had to be produced on a German PC with the language settings switched to “English” and so some words used in this image didn’t change with the rest.

Click the "Add" button...

## Step 2

Activate the control box "Don't detect my modem; I will select it from a list" and

click the "Next" button...

## Step 3

Click the "Have Disk..." button...

Enter here the drive and folder the p35modem.inf file is stored in. With the "Browse" button you can search the file, should you be uncertain as to where you stored it.

* we apologize, this document had to be produced on a German PC with the language settings switched to “English” and so some words used in this image didn’t change with the rest.

Click the "Open" button...

## Step 4

There are two types of modem available for each of the three mobile phones listed, one analog modem with the V.32 protocol and one ISDN modem supporting V.110.

The fast connection establishment is the advantage of the ISDN modem, but  you have to be sure that the opposite ISDN modem supports V.110 with a bitrate adaptation of 9600 bit/s.

Choose one of the modems and click the "Next" button...

## Step 5

Choose the serial interface (here COM1) your mobile (here: S35i) is connected to.

The connection between PC and S25 can be realized either via cable (RS232), or via infrared (IrDA). For the infrared connection, you have to choose the the virtual Infrared COM port (here the "COM4 (IrCOMM)").

Click the "Next" button...

## Step 6

Click the "Finish" button...

## Step 7

*we apologize, this document had to be produced on a German PC with the language settings switched to “English” and so some words used in this image didn’t change with the rest.

Now the modem has been successfully installed. (here: SIEMENS S35i (Analog V.32))

Click the "Close" button...